| Improvements |  |
| --- | --- |
| Name of Property: | Sheridan Motors GMC Truck Center |
| Property Type: | Full Service Auto Dealership |
| Property Sub-Type: | Light Industrial – Shop/Office |
| Occupancy: | Tenant Occupied (Sheridan Motors) |
| General |  |
| Number of Buildings: | 1 |
| Stories: | 1 |
| Year Built: | 2024 |
| Year Remodeled: | NA |
| Basement: | None |
| Construction Type: | Pre-engineered Steel |
| Construction Quality: | Very Good |
| Condition: | New |
| Gross Building Area: | 11,174 sf |
| Main Floor Area: | 11,174 sf |
| % of Office: | 45% |
| % of Shop: | 55% |
| Sidewall Height: | 18’ – 22’ |
| Exterior |  |
| Foundation/Floor: | Poured footings and foundation; slab-on-grade |
| Roof: | TPO Membrane |
| Exterior Walls: | The exterior will feature a combination of aluminum composite material, EIFS, CMU, and fenestration. |
| Interior Finishes |  |
| Ceilings: | The showroom/offices and service counter areas are to have a combination of acoustic ceiling tile and painted drywall. The service area and parts storage areas are to have exposed ceilings with painted metal rafters. |
| Flooring: | There is to be a combination of porcelain tile, carpeting, carpet tile, and polished concrete. |
| Walls: | A combination of textured/painted drywall, storefront glass, perforated metal, aluminum laminate, and porcelain tile. |
| Doors: | The storefront entrance is to feature aluminum framed storefront doors. Other exterior doors are to be hollow metal. Interior doors are to be a combination of solid wood, metal framed w/glass, and hollow metal. Each service bay is to have (1) 12’ x 12’ overhead door for a total of (5) overhead doors. There are to be (2) 10’ x 10’ overhead doors to the receiving area along the south elevation. |
| Windows: | Various sizes of double paned metal framed windows. |
| Mechanical Systems |  |
| Electrical: | Assumed to be adequate |
| Heating/Cooling: | HVAC heating and cooling with (2) rooftop units, (1) electric unit heater, and (6) gas fired unit heaters. |
| Plumbing: | The men’s restroom is to feature (1 ADA toilet / 2 urinals / 2 sinks) the women’s restroom is to have (1 ADA toilet / 2 toilets / 2 sinks). The service and parts area restroom is to have (1 toilet / 1 sink). The service area is to feature a double sink and washer and dryer hookup. The customer lounge is to feature a kitchenette having one sink. Additionally, the service area is to be plumbed with air. |
| Sprinkler: | None |
| Elevators: | None |
| Site Improvements |  |
| Asphalt: | Asphalt paved parking lot along the west side of the building featuring 45 spaces. As previously mentioned, the western approximately ¾ of the site was improved with an asphalt paved parking lot at the time of the sale. |
| Concrete: | Concrete city sidewalks on the west of the site along Wrench Drive and along the south side of the site along North Main Street. There is to be a concrete apron along the south and east elevations of the building in front of the overhead doors. Additionally, there is concrete sidewalks to the entrance and concrete curbing along the parking lot. |
| Landscaping: | It is assumed there will be good quality landscaping similar to the current landscaping along the west and south sides of the site featuring bushes, trees, irrigated sod and landscape rock. |
| General Comments |  |
| This is to be a very good quality full service automobile dealership. The interior finishes are to be of very good quality having high end finishes, fitting the GMC brand. |  |